| JOB/TASK NAME: Load Bark into the Cart with Tractor |  |  |  | PAGE 1 OF 1 |  | DATE: 11/4/14 | NEW REVISED |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EMPLOYEE(S)/POSITION(S) PERFORMING THE JOB: Kurt Higbee |  |  |  | SUPERVISOR(S): |  | ANALYSIS BY: Kurt Higbee |  |
| PLANT/LOCATION: Facilities Yard |  | DEPARTMENT(S): Grounds |  | SHIFT (if applicable): |  | APPROVED BY: Chris Gale |  |
| PERSONAL PROTECTIVE EQUIPMENT: Safety Glasses |  |  |  |  |  |  |  |
| TRAINING REQUIREMENTS: |  |  |  |  |  |  |  |
| JOB STEPS |  |  | POTENTIAL HAZARDS |  | ACTION/PROCEDURE TO CONTROL OR ELIMINATE |  |  |
| 1 | Setting the brake on the cart |  | Could run into cart Backing Hazard |  | Park cart on one side of bark bin Set Brake Use tractor on other side of bin Start tractor and set brake on cart |  |  |
| 2 | Activating hydraulics |  | Running at low throttle makes tractor respond lower |  | Set throttle at full throttle Use bucket handle set at level Lower the bucket Drive into pile of bark |  |  |
| 3 | Lifting bucket high enough |  | Having bucket too low when loading could run into cart causing it to tip or maybe hit a tire causing a flat |  | Curl bucket back until it stops Lift bucket handle set at level Lower the bucket Drive into pile of bark |  |  |
| 4 | Filling cart bed |  | Overloading cart could cause tire damage and unstable when driving When dumping bark the bark chips could get into eyes |  | Drive forward towards carts Stop when bucket is over cart bed Fill bed on the cart to the height of the bed rails Wear safety glasses |  |  |
| 5 | Raising the bucket to clear cart bed |  | Raising the bucket too high and driving is a potential for tipping, especially on unleveled ground |  | After dumping roll back bucket Back up tractor with bucket raised just high enough to clear cart bed. |  |  |

Job Safety Analysis Form