| JOB/TASK NAME: Mouse Dropping Removal |  |  |  | PAGE 1 OF 1 |  | DATE: 2/17/15 | NEW REVISED |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EMPLOYEE(S)/POSITION(S) PERFORMING THE JOB: HVAC |  |  |  | SUPERVISOR(S): |  | ANALYSIS BY: Ryan Glazier |  |
| PLANT/LOCATION: HVAC |  | DEPARTMENT(S): Facilities |  | SHIFT (if applicable): |  | APPROVED BY: |  |
| PERSONAL PROTECTIVE EQUIPMENT: Goggles, Gloves, Coveralls |  |  |  |  |  |  |  |
| TRAINING REQUIREMENTS: |  |  |  |  |  |  |  |
| JOB STEPS |  |  | POTENTIAL HAZARDS |  | ACTION/PROCEDURE TO CONTROL OR ELIMINATE |  |  |
| 1 | Section off area. Put on protective equipment. |  | Slip or trip hazards |  | Wear non slip safety shoes, Hepa ploo respirator, googles, gloves, protective coveralls. |  |  |
| 2 | Mix disinfectant solution. (1:10 bleach/water) in spray bottle, label contents of spray bottle. |  | Splash hazard |  | Use a funnel to pour bleach, clean up any spills. |  |  |
| 3 | Spray droppings gently with bleach solution, make sure they are completely soaked. Allow to sit for 15 minutes. |  | Toxic dust, infectious waste, blood borne pathogens. |  | Wear PPE, 15 minute soak with 10% bleach to destroy viruses. |  |  |
| 4 | Remove soaked material using gloves, vacuum leftover materials with a hepa filter vacuum cleaner. |  | Toxic dust, infectious waste blood borne pathogens |  | Wear PPE |  |  |
| 5 | Using 2 trash bags, place material into bag 1 and tie shut. While still wearing gloves, remove goggles, mask and protective clothing and place into bag 2, seal bag and dispose |  | Toxic dust, infectious waste |  | Wear PPE, Follow procedure established in step 5. |  |  |

Job Safety Analysis Form