| Job Safety Analysis Form JOB/TASK NAME: Preparing Boilers for Internal Inspections & Season |  |  |  | PAGE 1 OF 2 |  | DATE: 12/10/14 | NEW REVISED |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EMPLOYEE(S)/POSITION(S) PERFORMING THE JOB: Allen Syrett, Scott Rogers |  |  |  | SUPERVISOR(S): Wes Smith |  | ANALYSIS BY: |  |
| PLANT/LOCATION: Heat Plant |  | DEPARTMENT(S): Facilities |  | SHIFT (if applicable): Day |  | APPROVED BY: |  |
| PERSONAL PROTECTIVE EQUIPMENT: Safety Glasses, Gloves |  |  |  |  |  |  |  |
| TRAINING REQUIREMENTS: |  |  |  |  |  |  |  |
| JOB STEPS |  |  | POTENTIAL HAZARDS |  | ACTION/PROCEDURE TO CONTROL OR ELIMINATE |  |  |
| 1 | Make sure boilers and gas lines are locked out tagged out. |  | Boilers being Turned On or Electrical Shock |  | Lockout & Tagout the boilers & electrical panels & gas lines |  |  |
| 2 | Get proper PPE equipment tools needed for the job. |  | Flying Debris, Pinch Points, Cuts, etc. |  | Wear gloves, eye protection, any other PPE employee sees fir to wear, use proper tools |  |  |
| 3 | Drain all boilers after cool down. Open top drum and rinse drum out as it empties. When drained enough, open mud drum and continue rinsing until completely empty. Then take oil caps off sight glass lines and all other as needed. Open fire boxes if going in drums and fire box you need to get confined space permit |  | Passing out while in drum or fire box. Breathing bad air |  | Perform air test before entering. Have a second around to watch in case of accident or mishap |  |  |