| Job Safety Analysis Form JOB/TASK NAME: Welding |  |  |  | PAGE 1 OF 1 |  | DATE: 2/20/15 | NEW REVISED |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EMPLOYEE(S)/POSITION(S) PERFORMING THE JOB: Grounds |  |  |  | SUPERVISOR(S): Levi Reed |  | ANALYSIS BY: Jacob Carter |  |
| PLANT/LOCATION: Campus |  | DEPARTMENT(S): Grounds |  | SHIFT (if applicable): |  | APPROVED BY: Chris Gale |  |
| PERSONAL PROTECTIVE EQUIPMENT: Gloves, Welding Mask |  |  |  |  |  |  |  |
| TRAINING REQUIREMENTS: |  |  |  |  |  |  |  |
| JOB STEPS |  |  | POTENTIAL HAZARDS |  | ACTION/PROCEDURE TO CONTROL OR ELIMINATE |  |  |
| 1 | Clean off object to be welded |  | Sharp edges could cause lacerations to hands or body |  | Wear leather gloves |  |  |
| 2 | Spray weld primer onto anything to be welded (zinc spray) |  | Inhaling zinc fumes |  | Only use zinc spray in a well ventilated area, always spray away from face |  |  |
| 3 | Open gas until pressure is 25 psi on flow meter |  | None |  | None |  |  |
| 4 | Hook grounding clamp on object near welding area |  | Pinch points from clamp |  | Keep fingers and hands firmly on handle of clamp away from object and teeth of the clamp |  |  |
| 5 | Plug in welding machine |  | Frayed wire could cause electrical shock |  | Check over electrical wire for any worn insulation |  |  |
| 6 | Put on welding mask |  | Visor of a wrong grade design for the type of welding being conducted could cause damage to eyes |  | Check with manufacturer instruction manual for the types of jobs the visor is designed for |  |  |
| 7 | Conduct several test welds on scrap metal to achieve the proper wire feed speed |  | Hot surface and sparks could cause burns to hands or body. Ultraviolet light could cause eye damage. Welding fumes could cause lung cancer. |  | Wear leather gloves. Wear proper visor of a grade designed for the type of welding you are conducting. Weld only in well ventilated areas. Have a fire extinguisher on site at all times. |  |  |
| 8 | Touch wire to metal while moving in a smooth motion. Constantly holding down the torch trigger. |  | Same as above |  | Same as above |  |  |